Dynamic Ad Media Layer – Prior Art and Conflicting Patents Analysis

Date Captured: 2025-04-05

1. Prior Art Search Overview

A review of publicly known telecom and ad-tech systems reveals several types of related prior art:

| System/Patent Area | Description |
| --- | --- |
| Single DSP Media Layers | Systems where one DSP controls both app recommendation sourcing and engagement flow (e.g., Digital Turbine Ignite). |
| Carrier App Recommendation Systems | Carrier-owned recommendation engines, often basic, tied directly to app stores (e.g., Verizon AppFlash). |
| OOBE App Recommendation Flows | Basic out-of-box experiences offering a list of apps to install, with limited dynamic behavior (e.g., Samsung Smart Setup). |
| Ad Mediation Platforms | Systems managing multiple ad networks for banner ads or interstitial ads in mobile apps (e.g., AdMob Mediation). |

None of these known systems disclose a carrier-owned media layer that can dynamically source different content types from multiple DSPs independently with real-time reconfiguration, deferred engagement, and carrier-exclusive data optimization.

2. Closest Known Systems/Patents

| Patent/System | Short Description | Key Differences |
| --- | --- | --- |
| US20170366571A1 (Digital Turbine App Distribution) | Single DSP provides app installs via a media layer. | No multi-DSP dynamic arbitration. No carrier data optimization. Carrier does not own engagement. |
| US20200123249A1 (Samsung Setup Wizard App Recommendations) | Recommends apps at device setup. | No multi-DSP sourcing. No scoring or performance routing. No deferred engagement logic. |
| US20170293964A1 (Google AdMob Mediation) | Manages ad network waterfalls for ads in apps. | Ad-focused. No app recommendations at OOBE. No device engagement flow control. |
| US20190087976A1 (Verizon AppFlash) | Carrier-curated app launcher with recommendations. | Static sourcing. No DSP flexibility. Limited optimization using carrier-exclusive data. |

3. Novelty and Differentiation Table

| Feature | Dynamic Ad Media Layer | Prior Art |
| --- | --- | --- |
| Multi-DSP Modular Sourcing | ✅ Yes | ❌ No |
| Carrier-Owned Engagement Layer | ✅ Yes | ❌ No |
| DSP Role Assignment per Content Type | ✅ Yes | ❌ No |
| Deferred Engagement Scheduling | ✅ Yes | ❌ No |
| Carrier Data Optimization (Billing, App Usage) | ✅ Yes | ❌ No |
| Dynamic DSP Failover and Scoring | ✅ Yes | ❌ No |
| Engagement Mode Detection (Customer vs Store) | ✅ Yes | ❌ No |
| Flexible Presentation Formats (Grids, Video, Lists) | ✅ Yes | ❌ No |

4. Legal Language on Differentiation

The present invention significantly advances the state of the art by introducing a carrier-controlled, dynamic, multi-DSP media engagement layer that:

- Separates content sourcing duties across multiple independent DSPs.

- Dynamically reassigns content providers based on real-time performance metrics.

- Leverages proprietary carrier data to optimize app recommendation quality.

- Supports deferred engagement models tied to device setup conditions (e.g., store setup bypass).

- Operates under a carrier-first architecture where the media layer is fully owned and managed by the telecom operator, ensuring full control over customer engagement and monetization.

No known prior art discloses the full combination of these features, nor enables the carrier to dynamically arbitrate between multiple content providers while maintaining ownership of the engagement workflow.